IPVPN / NOC Exam Full Cheat Sheet

0) OSI model quick map

L1 Physical: bits, cable/fiber, link light, speed/duplex

L2 Data Link: frames, MAC, switching, VLAN, STP

L3 Network: packets, IP, routing, ICMP

L4 Transport: TCP/UDP, ports

L5 Session: sessions (rare in configs)

L6 Presentation: encryption/encoding (TLS conceptually)

L7 Application: DNS/HTTP/SSH/SNMP/DHCP/RDP etc.

1) Layer 2 (Data Link) — Switching, VLANs, loops

Ethernet — IEEE 802.3

Layer: L2 | Port: None | What: LAN framing + MAC delivery.

ARP — Address Resolution Protocol

Layer: Between L2/L3 | Port: None | What: Resolves IPv4 → MAC on LAN.

VLAN — Virtual Local Area Network

Layer: L2 | Port: None | What: Separates broadcast domains on a switch.

IEEE 802.1Q — VLAN Tagging

Layer: L2 | Port: None | What: Tags frames so trunks carry multiple VLANs.

DTP — Dynamic Trunking Protocol (Cisco)

Layer: L2 | Port: None | What: Negotiates trunking. Modes: dynamic auto/desirable/trunk/access.

VTP — VLAN Trunking Protocol (Cisco)

Layer: L2 | Port: None | What: Distributes VLAN database. Risk: revision number issues.

STP — Spanning Tree Protocol (IEEE 802.1D)

Layer: L2 | Port: None | What: Prevents switching loops by blocking paths.

RSTP — Rapid Spanning Tree Protocol (IEEE 802.1w)

Layer: L2 | Port: None | What: Faster convergence than STP.

LACP — Link Aggregation Control Protocol (IEEE 802.3ad / 802.1AX)

Layer: L2 | Port: None | What: Bundles links into one Port-Channel.

Core L2 show commands:

show mac address-table
show vlan brief
show interfaces trunk
show spanning-tree
show interfaces status
show interfaces counters errors

2) Layer 3 (Network) — IP, routing, MPLS

IPv4 — Internet Protocol version 4

Layer: L3 | Port: None | What: Addressing + routing between networks.

ICMP — Internet Control Message Protocol

Layer: L3 | Port: None | What: Ping + network errors (TTL exceeded, unreachable).

OSPF — Open Shortest Path First

Layer: L3 | Port: None | Transport: IP protocol 89 | What: Link-state routing (areas, DR/BDR).

EIGRP — Enhanced Interior Gateway Routing Protocol

Layer: L3 | Port: None | Transport: IP protocol 88 | What: Advanced distance-vector (DUAL).

MPLS — Multiprotocol Label Switching

Layer: 2.5 | Port: None | What: Label forwarding used in provider cores/IPVPN.

Core L3 show commands:

show ip interface brief
show ip route
show ip arp
show ip ospf neighbor
show ip ospf interface

3) Layer 4 (Transport)

TCP — Transmission Control Protocol

Layer: L4 | Port: N/A | What: Reliable, connection-based; handshake, window size, retransmissions.

UDP — User Datagram Protocol

Layer: L4 | Port: N/A | What: Connectionless, low overhead.

4) Layer 7 (Application) — services + ports

DNS — Domain Name System

Layer: L7 | Ports: UDP 53, TCP 53 | What: Name ↔ IP resolution.

DHCP — Dynamic Host Configuration Protocol

Layer: L7 | Ports: UDP 67/68 | What: Auto IP config (DORA).

HTTP — Hypertext Transfer Protocol

Layer: L7 | Port: TCP 80 | What: Web traffic (unencrypted).

HTTPS — Hypertext Transfer Protocol Secure (HTTP over TLS)

Layer: L7 (TLS conceptually L6) | Port: TCP 443 | What: Encrypted web traffic.

SSH — Secure Shell

Layer: L7 | Port: TCP 22 | What: Secure remote CLI.

Telnet — TELecommunication NETwork

Layer: L7 | Port: TCP 23 | What: Remote CLI (insecure plaintext).

SNMP — Simple Network Management Protocol

Layer: L7 | Ports: UDP 161/162 | What: Monitoring + traps.

NTP — Network Time Protocol

Layer: L7 | Port: UDP 123 | What: Time synchronization.

Syslog

Layer: L7 | Port: UDP 514 (common) | What: Central logging.

RIP — Routing Information Protocol

Layer: L7 | Port: UDP 520 | What: Distance-vector routing (hop count metric).

BGP — Border Gateway Protocol

Layer: L7 | Port: TCP 179 | What: Policy routing between ASes; common in ISP/MPLS VPN.

RDP — Remote Desktop Protocol

Layer: L7 | Ports: TCP 3389 (+ often UDP 3389) | What: Remote Windows GUI access.

5) ACL — Access Control Lists

ACL — Access Control List

Layer: Mainly L3/L4 (filters IP + protocol + ports)

Port: N/A (ACL is a filter, not a protocol)

Implicit deny at the end

Top-to-bottom processing; first match wins

Standard ACL: usually source IP only; Extended ACL: src/dst + protocol + ports

Apply inbound or outbound on an interface

Useful checks:

show access-lists
show ip interface
show run | include access-group

6) NAT / PAT

NAT — Network Address Translation (Layer: L3/L4 function | Port: N/A)

PAT — Port Address Translation / NAT Overload (Layer: L3/L4 function | Port: N/A)

Commands:

show ip nat translations
show ip nat statistics

7) Must-know special IPv4 ranges

10.0.0.0/8 Private

172.16.0.0/12 Private

192.168.0.0/16 Private

127.0.0.0/8 Loopback

169.254.0.0/16 APIPA (DHCP failed)

224.0.0.0/4 Multicast

0.0.0.0/0 Default route

Hosts = 2^(host bits) - 2 (except /31)

/30 = 4 IPs (2 usable), /31 = 2 usable (P2P)

8) Topology cheat sheet

Star LAN: PCs -> Switch -> Router

PCs -> Switch -> Router

Point-to-Point WAN: R1 -------- R2

R1 -------- R2

Redundant triangle (STP case):

SW1
  /  \
SW2--SW3

Hub-and-Spoke WAN: Branches -> HQ -> Internet/DC

Branches -> HQ -> Internet/DC

MPLS/IPVPN provider: CE--PE====P====PE--CE

CE--PE====P====PE--CE

9) Troubleshooting flow (layer-based)

L1: link? speed/duplex? errors?

L2: VLAN/trunk/STP/MAC table

L3: IP/mask/gateway, ARP, ping gateway

Routing: route table, OSPF neighbors FULL, default route

ACL/NAT: permitted? correct direction? translations?

DNS/App: DNS resolves? correct port open?